# FMP Unit 1 — Farm Mechanization and Equipment (ARS/NET Notes + MCQs)

# FARM MACHINERY & POWER — UNIT 1

# Farm Mechanization and Equipment

## Study Notes + MCQ Bank for ARS/NET (ASRB)

Sources: Kepner, Bainer & Barger — Principles of Farm Machinery; Government of India (Ministry of Agriculture & Farmers Welfare / PIB) data on mechanization; standard Indian FMP references (Jagdishwar Sahay, Ojha & Michael); NPTEL Farm Machinery courses. India-specific statistics verified from published sources as of July 2026.

# SECTION 1: STATUS OF FARM MECHANIZATION IN INDIA

## 1.1 Definition and Scope

Farm mechanization is the application of engineering and technology inputs (mechanical, electrical, and renewable power sources with matching implements) to perform agricultural operations in a timely and efficient manner, reducing drudgery and cost while raising productivity.

Benefits (frequently asked): timeliness of operations (saves 15–20% of inputs like seed and fertilizer), reduction of drudgery, precision in metering and placement, higher cropping intensity, reduction in cost of operation per unit output, and enabling conservation agriculture.

## 1.2 Farm Power Availability — Trends (memorize these figures)

| Year | Farm power availability (kW/ha) |
| --- | --- |
| 1951–52 | ~0.25 |
| 1975–76 | 0.48 |
| 2013–14 | 1.84 |
| 2018–19 | 2.49 |
| 2019–20 | 2.54 |
| 2021–22 | ~3.04 |
| Target 2030 | 4.0 |

- There is a strong positive correlation between farm power availability (kW/ha) and food-grain productivity (t/ha) across Indian states — Punjab and Haryana have the highest power availability and productivity.

- Overall level of farm mechanization in India: ~45–47%, against ~95% in the USA, ~75% in Brazil, and ~57–60% in China.

## 1.3 Shift in Power Sources (animate → mechanical)

- 1960–61: animate sources (human + draught animal) supplied ~93–97% of farm power.

- 2018–19 onward: mechanical + electrical sources supply >90% of total farm power.

- Approximate present shares of farm power: tractors ~47–50%, electric motors ~25–27%, diesel engines ~8–9%, draught animals ~5%, agricultural workers ~5%, power tillers ~1%.

- Tractor is the single largest source of farm power in India.

## 1.4 Operation-wise Mechanization Levels (approximate)

| Operation | Mechanization level |
| --- | --- |
| Soil working & seedbed preparation | ~40% |
| Seeding & planting | ~29–30% |
| Plant protection | ~34–35% |
| Irrigation | ~37% |
| Harvesting & threshing (wheat, rice) | 60–70% |
| Harvesting (other crops) | <15–20% |

Rice transplanting, cotton picking, sugarcane harvesting, and horticultural operations remain the least mechanized.

## 1.5 Tractor Industry

- India is the world’s largest tractor manufacturer and market; domestic sales crossed ~1.0 million units in 2025 (about one-third of global production).

- Major manufacturers: Mahindra & Mahindra (largest, ~40%+ share), TAFE, Sonalika (ITL), Escorts Kubota, John Deere India, New Holland.

- Most-sold power band in India: 31–40 hp and 41–50 hp categories.

- Tractor density is highest in the north-western states (Punjab, Haryana, western UP).

## 1.6 Government Programmes and Institutions

- SMAM (Sub-Mission on Agricultural Mechanization) — launched 2014–15; components include Custom Hiring Centres (CHCs), Hi-Tech Hubs, Farm Machinery Banks, subsidies of 40–50% (up to 80% for CHC/group ownership), training and testing.

- CRM Scheme (Promotion of Agricultural Mechanization for In-Situ Management of Crop Residue) — 2018; promotes Happy Seeder, Super SMS, mulcher, zero-till drill, balers to curb residue burning in Punjab, Haryana, UP, NCT Delhi.

- FARMS mobile app (Farm Machinery Solutions) — 2019 — Uberisation of custom hiring.

- Farm Machinery Training & Testing Institutes (FMTTIs) — 4 institutes:

  - Budni (Madhya Pradesh) — Central FMTTI, established 1955; tractor testing (national tractor test code).

  - Hisar (Haryana) — Northern Region FMTTI, 1963.

  - Garladinne, Anantapur (Andhra Pradesh) — Southern Region FMTTI, 1983.

  - Biswanath Chariali (Assam) — North-Eastern Region FMTTI, 1990.

- CIAE Bhopal (Central Institute of Agricultural Engineering, est. 1976, ICAR) — R&D on farm equipment; AICRP on Farm Implements and Machinery (FIM), AICRP on Ergonomics & Safety (ESA), AICRP on Utilization of Animal Energy.

- Dangerous Machines (Regulation) Act, 1983 — covers power threshers, chaff cutters, sugarcane crushers (operator safety).

# SECTION 2: TILLAGE AND LAND DEVELOPMENT EQUIPMENT

## 2.1 Tillage — Definitions

Tillage is the mechanical manipulation of soil to provide a favourable environment for crop growth.

- Primary tillage: initial, deeper opening of soil (12–30 cm or more) — MB plough, disc plough, chisel plough, subsoiler, rotary plough.

- Secondary tillage: shallower, refining operations (5–10 cm) — harrows, cultivators, levellers, pulverizers, rollers.

- Minimum tillage: least soil manipulation necessary; Zero/No-till: direct seeding into unprepared soil; Conservation tillage: ≥30% of surface covered by residue after planting; Strip/Rotary/Mulch tillage are other classes.

## 2.2 Mouldboard (MB) Plough

Performs cutting, lifting, inverting (complete inversion is its distinguishing action) and pulverizing the furrow slice. Good for burying weeds and trash.

Parts: share (penetrates and cuts), mouldboard (lifts and inverts), landside (absorbs side thrust; rear end has heel), frog (base to which all parts bolt), shin (cutting edge extension of MB), tail piece.

- Share types: slip share, slip-nose share, shin share, bar-point share (most common where abrasive soils rapidly wear points; point can be extended).

- Share materials: chilled cast iron or soft-centre steel (outer hard layers over soft core).

- Mouldboard types: General purpose (medium pitch, most common), Stubble (short, abruptly curved — maximum pulverization), Sod/Breaker (long, gently curved — complete inversion without breaking slice), Slat (sticky soils), High-speed (slender, for 6–8 km/h+).

- Clearance/suction: Vertical suction (down suction) helps penetration (typically 3–5 mm for tractor ploughs); Horizontal suction (side suction/land suction) helps maintain furrow width (typically 3–10 mm).

- Plough size = width of cut of one bottom (e.g., 2 × 30 cm = 2 bottoms of 30 cm each).

- Types of ploughing: gathering (clockwise, starts at centre), casting (anticlockwise, starts at sides), continuous (one-way/reversible plough — no back furrows or dead furrows, ideal for hillsides and irrigated lands).

- Specific/unit draft = draft ÷ furrow cross-sectional area (kgf/cm² or N/cm²). Typical unit draft: light sandy soils ~0.2–0.4 kgf/cm², heavy clays 0.7–1.4 kgf/cm² (Kepner: draft varies roughly with speed as D = a + bV²).

## 2.3 Disc Plough

Cuts, partially inverts and mixes the soil; no landside in standard disc plough (side thrust taken by rear furrow wheel).

- Standard disc plough: individually mounted concave discs, 60–90 cm diameter, 6–10 mm thick.

  - Disc angle: angle between disc face plane and direction of travel = 42–45° (increasing disc angle → wider cut, better penetration in hard soil).

  - Tilt angle: angle between disc plane and vertical = 15–25° (increases penetration in hard soils).

  - Penetration depends on weight (unlike suction in MB plough) — hence discs carry added ballast; typical weight 180–540 kg per disc bottom equivalent.

- Vertical disc plough (harrow plough/one-way disc): discs on a common gang axle, 50–60 cm discs, shallow (8–10 cm) — wheatland plough.

- Advantages over MB: works in sticky, hard, stony, rooty soils and in soils where scouring is a problem; rolls over obstacles.

- Disadvantages: incomplete inversion, leaves cloddy surface, high weight requirement, rear furrow wheel needed.

## 2.4 Chisel Plough, Subsoiler, Rotavator

- Chisel plough: breaks hard pan/soil to 30–45 cm with narrow curved shanks; no inversion; row spacing of shanks 30 cm typical.

- Subsoiler: heavier standard, shatters impervious hardpan at 45–90 cm depth; requires 60–100 hp; winged subsoilers increase shatter width; mole plough forms drainage channels (mole drains) in clay soils.

- Rotavator (rotary tiller): PTO-driven blades (L-type most common in India for trashy conditions; also C and J types) on a horizontal shaft; cuts and pulverizes in a single pass — seedbed in one operation; negative draft (bite length = forward travel per blade strike) determines pulverization; power consumption high; excellent for puddling.

  - Blade tip speed 4–9 m/s; rotor speed 200–350 rpm typical.

- Puddlers/puddling: breaking soil aggregates in standing water to create impervious layer for paddy — cage wheel + rotavator or peg-type puddler.

## 2.5 Secondary Tillage & Land Development

- Disc harrow: gangs of concave discs (35–70 cm); types — single action, tandem (double action), offset (works offset to tractor centre-line — orchards), plough harrow. Gang angle 15–30°. Penetration increased by increasing gang angle/adding weight.

- Cultivators: rigid tine, spring-loaded tine (stony soils — trip and reset), duck-foot (sweep) cultivator for shallow weed-kill and moisture conservation.

- Blade harrow (bakhar): indigenous implement — cuts weeds below surface without inversion, retains residue mulch.

- Levellers/scrapers: bund former, ridger (ridging body = two opposed mouldboards), laser-guided land leveller (±2 cm accuracy; saves 20–30% irrigation water; components: laser transmitter, receiver, control box, hydraulic valve, drag bucket).

- Rollers/pulverizers: clod crushers, plankers for compacting/firming seedbed.

## 2.6 Draft, Pull and Power (working definitions used throughout Unit 1)

- Pull = total force exerted by tractor on implement (along line of hitch).

- Draft = horizontal component of pull, in direction of travel: Draft = Pull × cos θ.

- Unit draft = draft per unit cross-sectional area of furrow.

- Drawbar power (kW) = Draft (kN) × Speed (m/s); in field units: DBP (hp) = (Draft, kgf × Speed, m/s)/75.

- Typical speed of primary tillage 4–6 km/h; secondary 5–8 km/h.

# SECTION 3: SOWING, PLANTING AND TRANSPLANTING EQUIPMENT

## 3.1 Methods of Sowing

Broadcasting, drilling (random placement in rows), precision planting (single seeds at uniform intervals), dibbling (placing seeds in holes), hill dropping (groups of seeds at intervals), check-row planting (hills in both directions — cross-cultivation possible), transplanting.

## 3.2 Seed Drill — Functional Components

Frame → hopper → seed metering mechanism → seed tube → furrow opener → covering device (+ transport/drive wheels, ground-drive transmission).

Functions: meter seed, open furrow, place seed at uniform depth, cover and compact.

## 3.3 Seed Metering Mechanisms (Kepner §11.6)

| Type | Working | Use |
| --- | --- | --- |
| Fluted roller (fluted wheel) | Adjustable exposed length of flutes carries seed; positive, force-feed | Most common on Indian seed drills (wheat etc.) |
| Internal double-run | Double-faced wheel (large/small side) inside casing | Grain drills; large + small seeds |
| Cup feed | Cups on vertical disc pick seed from hopper base | UK-type drills, paddy |
| Cell/plate type — edge drop, flat drop, hill drop | Cells on rotating plate accommodate single seed/groups | Row-crop planters (maize, cotton) |
| Vacuum/pneumatic disc | Seed held on perforated disc by suction | Precision planters — vegetables, maize |
| Inclined plate | Cells on inclined rotating plate; gentle | Groundnut (minimises seed damage) |
| Stationary opening + agitator | Adjustable orifice with agitator | Broadcasters, vegetable seeders |

- Fluted-wheel metering is more positive/uniform than double-run (Kepner §11.6); seed rate changed by shifting flute exposure.

- Seed damage highest with fluted roller for large fragile seeds; inclined plate/pneumatic types minimise damage.

## 3.4 Furrow Openers (Kepner §11.7)

- Fixed type: shoe (trashy, hard soils — India standard), shovel/hoe (stony), full runner (mellow trash-free soils, maintains depth — maize planters), stub runner.

- Rotating type: single-disc (cuts trash, minimum-till), double-disc (precise seed placement, sticky soils won’t scour fixed openers).

- Covering devices: chains, press wheels, paired covering discs.

## 3.5 Planters and Special Drills

- Row-crop planter: hill-drop or drill-plant seed plates; plate types — edge drop, flat drop, hill drop; cell fill affected by plate speed (keep plate peripheral speed low for uniform fill).

- Precision (pneumatic/vacuum) planter: near-100% singulation; vacuum 40–60 mbar typical; used for maize, sunflower, cotton, vegetables.

- Seed-cum-fertilizer drill: separate hoppers and metering for seed and fertilizer; fertilizer placed 5 cm below and 5 cm beside the seed (band placement).

- Zero-till drill: inverted-T openers sow directly into unploughed stubble field; saves ~60–80% energy of conventional sowing of wheat after rice.

- Happy Seeder: zero-till drill + flail straw-management rotor — sows wheat into combine-harvested paddy residue (in-situ residue management); Super Seeder ploughs residue in with rotavator + sows in one pass.

- Strip-till drill, Turbo seeder, roto-till drill — related conservation machines.

- Calibration of seed drill: jack the drive wheel, mark and rotate N revolutions equal to a known area: Area covered = π × D × N × W × n (D = wheel dia, W = row spacing, n = no. of rows); collect and weigh seed from each tube; seed rate (kg/ha) = collected mass × 10,000/area. (Full procedure — Unit 3, but definition asked in Unit 1 too.)

## 3.6 Transplanters

- Self-propelled rice transplanter: uses mat-type nursery (seedlings raised on soil layer over polythene in trays); fixed row spacing 238 mm (≈23–24 cm) in Japanese machines (also 30 cm models); hill spacing adjustable 12–21 cm; 2–4 seedlings/hill; finger/fork type picking mechanism with four-bar/planetary linkage; walk-behind (4–6 rows) and riding (6–8 rows) types; field capacity ≈ 0.15–0.2 ha/h (6-row).

- Vegetable transplanters (semi-automatic): operator feeds seedlings/plug seedlings into rotating cups/fingers; pro-tray (plug) seedlings essential for automatic types.

## 3.7 Special Planting Equipment

- Potato planter: cup-belt type (vertical belt with cups) and picker-wheel type (pins pierce tubers — some damage); plants on ridges; automatic and semi-automatic.

- Sugarcane cutter-planter: cuts whole cane into setts (2–3 bud) in-furrow, places, applies fertilizer + insecticide and covers in one pass; ring-pit digger for ring-pit planting; ratoon management device (RMD) — stubble shaving, off-barring, fertilizer placement in ratoon crop.

- Cotton planter: pneumatic or inclined-plate, delinted seed preferred.

- Groundnut planter: inclined-plate (low seed damage).

# SECTION 4: FERTILIZER APPLICATION EQUIPMENT

- Broadcasters: centrifugal (spinning-disc) spreader — swath 6–12 m, distribution pattern triangular, overlap needed; pendulum (oscillating spout); aircraft.

- Band placement: fertilizer attachments on seed drills/planters; star-wheel, fluted roller, auger, or agitator feed mechanisms for granular fertilizer (Kepner ch. 13).

- Fertilizer metering is difficult because fertilizers are hygroscopic and corrosive — hoppers need agitators and corrosion-resistant parts (stainless/plastic).

- Deep placement applicators for urea briquettes/USG (urea super granules) in paddy.

- Liquid fertilizer/anhydrous ammonia applicators — injection behind knife openers; fertigation through drip.

- Manure/FYM spreaders — box with apron conveyor + beaters.

# SECTION 5: INTER-CULTIVATION EQUIPMENT

- Objectives: weed control (biggest benefit — Kepner ch. 12), soil mulch, aeration, earthing-up.

- Hand tools: khurpi, hand hoe, wheel hoe (single/double wheel with interchangeable sweeps/tines; capacity ~0.015–0.02 ha/h), peg/spike weeder.

- Cono-weeder (paddy, between rows in puddled field — two conical rotors with smooth and serrated blades).

- Dry-land weeders, blade hoe, sweep cultivator (V-sweeps cut weeds below surface with least soil disturbance and moisture loss).

- Power weeders: engine-operated rotary weeders (1–2 hp petrol/kerosene) for row crops; tractor-mounted row-crop cultivators with shields to protect young plants.

- Rotary hoe: high-speed blind cultivation before/after crop emergence (Kepner ch. 12).

- Timing: cultivate shallow (3–5 cm) to avoid root pruning; flame cultivation and thinning (Kepner ch. 12) — selective flaming when cotton stems are hardened.

# SECTION 6: PLANT PROTECTION EQUIPMENT

## 6.1 Sprayer Functions & Classification

Functions: atomize spray liquid into effective droplet size, distribute uniformly, regulate application rate, direct spray to target.

By operation/pressure:

| Sprayer | Pressure | Notes |
| --- | --- | --- |
| Hand/hydraulic knapsack | 1–4 bar (low) | Lever-operated piston/diaphragm pump, 8–16 L tank |
| Hand compression | 3–7 bar | Pressurized tank |
| Foot/pedal sprayer | up to 17–21 bar | Stationary, hose + lance |
| Rocker sprayer | 14–28 bar | Tall trees with extension lance |
| Power knapsack mist blower | Air blast (low liquid pressure) | 35–100 μm droplets, air velocity ~100–200 m/s at outlet |
| Tractor-mounted boom sprayer | 2–7 bar (field crops) | Hydraulic nozzles on boom, 300–500 L tank |
| Air-blast/air-carrier orchard sprayer | 10–40 bar liquid + air stream | Air displaces & carries droplets into canopy |
| ULV/CDA (spinning disc) | — | Ultra-low volume < 5 L/ha, controlled droplet 60–120 μm |

Application volumes: High volume >400 L/ha (field) / >1000 L/ha (trees); Low volume 5–400 L/ha; Ultra-low volume (ULV) <5 L/ha.

## 6.2 Nozzles and Droplets

- Hollow-cone nozzle: ring-shaped deposit; insecticide/fungicide on foliage; swirl (whirl) plate gives tangential velocity.

- Solid-cone: full circular pattern.

- Flat-fan: tapered-edge elliptical pattern; herbicide spraying on flat surfaces; overlap 30–50%.

- Flood (deflector/anvil), solid-stream (gun), air-induction (AI) nozzles (coarse, anti-drift).

- Discharge q ∝ √P: doubling pressure increases discharge by √2 (~41%); droplet size decreases with pressure.

- Droplet size expressed as VMD (volume median diameter, μm); NMD (number median diameter); Uniformity coefficient = VMD/NMD (closer to 1 = more uniform).

- Optimum droplet sizes: flying insects 10–50 μm, foliage insects 30–50 μm, fungicides on foliage 30–150 μm, herbicides 250–500 μm (drift control).

- Drift-prone droplets: <100–150 μm.

## 6.3 Pumps and Agitation (Kepner ch. 21)

- Positive-displacement pumps: piston/plunger (high pressure, orchard), roller (vane) pump — most common on low-medium pressure field sprayers, gear, diaphragm (handles abrasives/wettable powders well).

- Non-positive: centrifugal (high volume, air-carrier sprayers).

- Agitation (for suspensions/emulsions): mechanical — paddles at 100–200 rpm near tank bottom (Kepner eq. 21.1: minimum paddle peripheral speed rises with liquid depth); paddle tip speeds >500 fpm cause foaming; hydraulic (jet) agitation — part of pump output returned through solid-stream nozzles aimed 30° above horizontal near tank bottom; most efficient agitation at low pressures; mechanical agitation more economical for high-pressure orchard sprayers (Kepner §21.15–21.16).

- Pressure regulator/relief valve, line strainers (50–100 mesh), cut-off valve are essential circuit elements.

## 6.4 Dusters (Kepner ch. 21)

- Apply dry dust (5–25% a.i.); components: hopper with agitator, feed control, fan/blower (rotary), delivery spouts.

- Types: hand rotary (fan) duster, bellows duster, knapsack power duster, tractor-mounted.

- Advantages: no water, light; Disadvantages: high drift, poor deposit/retention (needs dew), lower effectiveness than sprays.

## 6.5 Drones (basic Unit 1 level)

## Agricultural spraying drones (10–16 L tanks typical) with hollow-cone/flat-fan or centrifugal nozzles; 5–7× faster than knapsack; ULV rates 10–25 L/ha (some sources classify as low-volume); flight height 1.5–3 m above canopy; governed in India by Drone Rules 2021; Kisan Drones promoted with SMAM subsidy; SOPs issued by DA&FW for pesticide applicat

# SECTION 7: HARVESTING, MOWING, THRESHING, CHAFF CUTTING AND BALING

## 7.1 Mowers (Kepner ch. 14)

Cutter-bar (reciprocating) mower — parts: cutter bar, knife (sections riveted to knife back), guards (fingers) with ledger plates, wearing plates, knife clips, grass board & stick, shoes (inner/outer), pitman, flywheel.

- Pitch of knife sections and guards = 76.2 mm (3 in.); knife stroke = 76.2 mm (one section pitch).

- Knife velocity: crank speed 800–1000 rpm; average knife speed ≈ 2 × stroke × rpm.

- Registration: knife section centre coincides with guard centre at each end of stroke — adjusted by shifting cutter bar in/out.

- Alignment (lead): cutter bar given a forward lead of ~2% of bar length (≈2 cm per metre) to compensate backward deflection during cutting.

- Cutting is by shear between section edge and ledger plate.

- Rotary mowers/flail mowers — impact cutting; forage flail harvester.

## 7.2 Reapers and Windrowers

- Vertical conveyor reaper windrower (VCR): cutter bar + star wheels + vertical conveyor belts deliver harvested crop in a windrow at one side; 3–5 hp walk-behind (paddy/wheat), 0.15–0.2 ha/h.

- Reaper-binder: reaps and ties bundles with twine.

- Swather/windrower for large-area cereal/forage harvest.

## 7.3 Threshers

Threshing = detaching grain from ear head/panicle/pod, mostly by impact (≈80%), plus rubbing and squeezing actions between cylinder and concave.

Cylinder types (Kepner §17.4, Indian classification):

| Type | Features / crops |
| --- | --- |
| Spike (peg) tooth | Spikes on drum vs. spikes on concave; vigorous — wheat (desi threshers); breaks straw into bhusa |
| Rasp bar | Corrugated bars; most universal — combines; gentle on straw, tolerates green weeds |
| Angle bar | Rubber-faced helical bars — clover, alfalfa (minimal seed damage) |
| Wire loop | Loops on drum — paddy, ragi (drummy/pedal threshers) |
| Hammer mill type | Beaters — hard-to-thresh, makes fine bhusa (e.g., syndicator) |
| Axial flow | Crop moves axially around cylinder (spiral) — paddy thresher (IRRI design); high capacity, less grain damage |

- Cylinder peripheral speed (thumb rule): wheat 25–30 m/s (5000–6000 fpm — Kepner), paddy 12–18 m/s (wire loop), maize sheller 9–12 m/s; peripheral speed governs threshing effectiveness more than diameter (a given peripheral speed threshes similarly regardless of drum size).

- Concave clearance decreases from front (entry) to rear; smaller clearance & higher speed → better threshing but more grain breakage and straw break-up.

- Threshing effectiveness ↑ with cylinder speed, ↓ with higher feed rate, ↑ with lower moisture.

- Thresher losses: unthreshed (visible) loss, blown/sieve loss, spilled loss, broken (invisible/quality) loss.

- Safety: feeding chute length per IS 9020 (safe feeding); Dangerous Machines (Regulation) Act 1983.

- Components of a complete thresher: feeding unit → threshing cylinder & concave → separating (straw walkers/louvers) → cleaning (aspirator blower + oscillating sieves) → grain elevator/bagging.

## 7.4 Combine Harvester (Kepner ch. 17)

Performs cutting/gathering (header, reel, cutter bar, auger) → feeding (feeder conveyor) → threshing (cylinder–concave) → separation (straw walkers or rotors) → cleaning (chaffer, sieve, cleaning fan) → grain handling (elevators, tank) in one machine.

- Reel index (reel peripheral speed ÷ forward speed) = 1.25–1.5; reel axis ahead of cutter bar, height ~ centre of gravity of crop.

- Cylinder–concave clearance: more at entry, less at exit; wheat ~ 10–20 mm front, 3–10 mm rear (adjust per crop).

- Types: tractor-drawn (PTO), self-propelled wheat/paddy combines (most common in India, 3.6–4.3 m cutter bar), track combines for wet paddy fields.

- Header loss (shatter + cutter bar + lodged) is usually the largest loss component; measured by frame-count method.

- Straw combine/Super SMS (straw management system): chops and spreads straw behind combine — mandatory fitment on combines in Punjab/Haryana for residue management.

## 7.5 Mowing/Forage: Chaff Cutter, Forage Harvester

- Chaff cutter: hand/power operated; feeding rolls meter stalks to a flywheel-mounted knives (flywheel type) or cylinder-type cutterhead; cut length set by feed-roll speed and number of knives; power chaff cutters 2–5 hp / 0.5–2 t/h; safety per IS 7898 (feeding system with warning device) — commonly asked cause of hand injuries, covered under Dangerous Machines Act.

- Forage (flail/precision-chop) harvester: cuts, chops (theoretical length of cut = feed per knife), and blows chop into trailer (Kepner ch. 15).

## 7.6 Baling (Kepner ch. 14)

- Rectangular (square) baler: pickup → auger/packer → plunger (crank-driven, ~65–90 strokes/min) compresses charge in bale chamber → knotter (Deering/Appleby type knotter — billhook, twine disc, wiper arm) ties twine when bale reaches set length (star/metering wheel); bale density adjusted by tension rails; bale ~ 35–125 kg.

- Round baler: belts/rollers roll windrow into cylindrical bale (variable or fixed chamber), net/twine wrap; better weathering (rain sheds off).

- Straw balers behind combines (with pickup) central to residue management in NW India.

- Hay: mowing → conditioning (crusher/crimper rolls speed drying) → raking into windrows → baling at 18–22% moisture (loose stack ~25%; baled hay stored ≤ ~20% to avoid heating/mould).

## 7.7 Root/Tuber, Cotton, Groundnut, Sugarcane Harvesters (Kepner chs. 19–20 + Indian machines)

- Potato digger: elevator digger — share lifts ridge onto agitated rod-link chain conveyor that sifts soil; spinner/blade diggers simpler; potato combine adds picking table.

- Groundnut digger-shaker/inverter: blade cuts taproot below pods, lifts and inverts vines for field curing; thresher/pod stripper separates pods.

- Cotton:

  - Spindle picker (selective): tapered barbed rotating spindles wind seed cotton from open bolls; moistened pads; doffer removes cotton; multiple pickings possible.

  - Stripper (non-selective): finger/brush strips entire plant — open + unopened bolls, once-over harvest; suits compact/storm-proof varieties.

- Sugarcane harvester (chopper type): base cutter (counter-rotating discs) → feed rollers → chopping into billets (25–30 cm) → primary/secondary extractor fans remove trash → elevator to infield trailer; whole-stalk harvesters cut and windrow. Sugarcane stubble shaver, detrasher; sugarcane crushing period equipment: none (processing).

- Fruit/orchard: trunk/limb shakers with catching frames (nuts, some fruits), hydraulic elevating platforms/ladders, pneumatic secateurs; coconut/areca: mechanical palm-climbing devices (sit-and-climb type), telescopic harvesters; cashew: manual + shellers (processing).

- Vegetable crops: seedling transplanters, precision seeders, onion/garlic diggers, carrot harvesters (top-lifting).

# SECTION 8: ANIMAL-DRAWN EQUIPMENT (still asked in ARS/NET)

- Indigenous (desi) plough: wooden body, iron share point — opens V-shaped furrow, no inversion; draft ~ 40–60 kgf for a pair of bullocks.

- Improved animal-drawn: MB plough (Victory plough), bund former, blade harrow (bakhar), seed-cum-fertilizer drills (2–3 row), bullock-drawn planter, olpad thresher (disc harrow-like — wheat).

- A pair of good bullocks develops about 0.75–1.0 hp total (each ~0.5 hp sustained; can exert draft ≈ 1/10th of body weight).

- Yokes: neck/shoulder yoke conventional; improved (collar, three-pad) yokes raise draught efficiency and welfare.

# SECTION 9: ONE-PAGE FORMULA & FACT SHEET (rapid revision)

| Item | Value/Formula |
| --- | --- |
| Farm power availability (2021–22 → target) | ~3.04 → 4.0 kW/ha (2030) |
| Overall mechanization level, India | ~45–47% |
| Draft | Pull × cos θ |
| Drawbar power | Draft (kN) × speed (m/s) = kW |
| Unit draft | Draft ÷ furrow area (W×d per bottom × n) |
| Disc plough disc/tilt angle | 42–45° / 15–25° |
| MB plough share suction | vertical (penetration), horizontal (width) |
| Knife section pitch (mower) | 76.2 mm; lead of cutter bar ≈ 2% of length |
| Cylinder peripheral speed — wheat | 25–30 m/s (5000–6000 fpm) |
| Reel index (combine) | 1.25–1.5 |
| Nozzle discharge vs pressure | q ∝ √P |
| ULV rate | < 5 L/ha |
| Droplet uniformity | VMD/NMD → 1 |
| Rice transplanter row spacing | 238 mm (fixed, mat type) |
| Seed & fertilizer band placement | 5 cm below × 5 cm beside seed |
| Happy Seeder purpose | wheat sowing into paddy residue (zero till + straw handling) |
| Hydraulic agitation nozzles | aimed 30° above horizontal (Kepner §21.16) |
| Mechanical agitator paddles | 100–200 rpm; tip speed <500 fpm (foaming) |
| FMTTI HQ / oldest | Budni, MP (1955) |
| Thresher safety / chaff cutter safety | IS 9020 / IS 7898; Dangerous Machines Act 1983 |
| SMAM launch | 2014–15 |
| Tractor sales India 2025 | ~1.0 million units (world’s largest) |

# SECTION 10: MCQ QUESTION BANK (60 questions, ARS/NET pattern)

Q1. The average farm power availability in India in 2021–22 was approximately: (a) 1.84 kW/ha (b) 2.02 kW/ha (c) 3.04 kW/ha (d) 4.0 kW/ha

Q2. The Government of India target for farm power availability by 2030 is: (a) 3.5 kW/ha (b) 4.0 kW/ha (c) 4.5 kW/ha (d) 5.0 kW/ha

Q3. The single largest source of farm power in India at present is: (a) Electric motors (b) Diesel engines (c) Tractors (d) Draught animals

Q4. Overall level of farm mechanization in India is about: (a) 25–30% (b) 45–47% (c) 60–65% (d) 75–80%

Q5. SMAM was launched in: (a) 2010–11 (b) 2014–15 (c) 2016–17 (d) 2018–19

Q6. The Central Farm Machinery Training and Testing Institute is located at: (a) Hisar (b) Budni (c) Anantapur (d) Bhopal

Q7. Power threshers and chaff cutters are regulated for operator safety under: (a) Factories Act 1948 (b) Dangerous Machines (Regulation) Act 1983 (c) Insecticides Act 1968 (d) BIS Act 1986

Q8. Which operation has the LOWEST mechanization level in India? (a) Seedbed preparation (b) Wheat harvesting (c) Rice transplanting & cotton picking (d) Irrigation

Q9. Complete inversion of the furrow slice is the distinguishing action of: (a) Disc plough (b) Chisel plough (c) Mouldboard plough (d) Cultivator

Q10. The part of the MB plough that absorbs side thrust is the: (a) Frog (b) Share (c) Landside (d) Shin

Q11. Bar-point share is preferred in: (a) Sticky soils (b) Abrasive/stony soils causing rapid point wear (c) Sandy loams (d) Peat soils

Q12. A stubble-type mouldboard is characterized by: (a) Long, gently curved surface (b) Short, abruptly curved surface for maximum pulverization (c) Slats (d) Very slender shape for high speed

Q13. Vertical suction in an MB plough helps in: (a) Maintaining furrow width (b) Penetration to proper depth (c) Inversion (d) Reducing draft

Q14. In a standard disc plough, the disc angle usually ranges between: (a) 15–25° (b) 30–35° (c) 42–45° (d) 50–60°

Q15. Penetration in a disc plough is obtained mainly by: (a) Suction (b) Weight of the plough (c) Hydraulic down-pressure only (d) Sharpness of share

Q16. Which plough has no landside? (a) MB plough (b) Standard disc plough (c) Turn-wrest plough (d) Victory plough

Q17. Continuous ploughing without back furrows and dead furrows is achieved with: (a) Gathering method (b) Casting method (c) Two-way (reversible) plough (d) Tandem harrow

Q18. A subsoiler normally operates at a depth of: (a) 10–15 cm (b) 20–30 cm (c) 45–90 cm (d) 100–150 cm

Q19. The blade type most common on Indian rotavators for trashy conditions is: (a) C-type (b) J-type (c) L-type (d) Pick type

Q20. An offset disc harrow is most suited for: (a) Puddling (b) Orchards — working under tree canopies (c) Deep tillage (d) Ridging

Q21. Laser land levelling typically saves irrigation water to the extent of: (a) 5–10% (b) 20–30% (c) 50–60% (d) 70–80%

Q22. Draft of an implement equals: (a) Pull × sin θ (b) Pull × cos θ (c) Pull × tan θ (d) Pull ÷ cos θ

Q23. Which seed-metering mechanism is most common on Indian seed drills? (a) Cup feed (b) Internal double run (c) Fluted roller (d) Vacuum disc

Q24. Compared with the internal double-run feed, the fluted-wheel feed is: (a) Less positive in metering (b) More positive/uniform (c) Identical (d) Unsuitable for wheat

Q25. Which furrow opener is best for trash-covered, minimum-till fields? (a) Shoe (b) Full runner (c) Single-disc (d) Shovel

Q26. In a seed-cum-fertilizer drill, fertilizer is placed relative to seed at about: (a) Same level, mixed (b) 5 cm below and 5 cm to the side (c) 10 cm above (d) 15 cm below only

Q27. Happy Seeder is used for: (a) Transplanting rice (b) Direct drilling of wheat into combine-harvested paddy residue (c) Planting sugarcane setts (d) Sowing on ridges

Q28. Mat-type nursery is a prerequisite for: (a) Potato planter (b) Self-propelled rice transplanter (c) Sugarcane cutter planter (d) Pneumatic planter

Q29. Standard row-to-row spacing of Japanese-type mat rice transplanters is: (a) 150 mm (b) 200 mm (c) 238 mm (d) 300 mm

Q30. Sugarcane cutter-planter plants: (a) Whole cane (b) 2–3 bud setts cut in the furrow (c) Seedlings (d) Single-bud chips only

Q31. The picker-wheel type potato planter’s main drawback is: (a) Low capacity (b) Tuber damage by piercing picks (c) Cannot form ridges (d) High draft

Q32. Inclined-plate metering is preferred for groundnut because: (a) Higher seed rate (b) Minimum seed (kernel) damage (c) Cheaper (d) No calibration needed

Q33. Fertilizer hoppers require agitators and corrosion-resistant materials because fertilizers are: (a) Volatile (b) Hygroscopic and corrosive (c) Explosive (d) Oily

Q34. Spinning-disc broadcaster distributes fertilizer in a pattern best described as: (a) Uniform rectangular (b) Triangular, needing overlap (c) Ring shaped (d) Random

Q35. Cono-weeder is used in: (a) Dryland wheat (b) Puddled paddy between rows (c) Sugarcane earthing (d) Cotton topping

Q36. Sweep-type cultivators are favoured in dryland inter-culture because they: (a) Invert soil (b) Cut weeds below surface with least moisture loss (c) Work deep (d) Form ridges

Q37. For herbicide application on flat surfaces, the recommended nozzle is: (a) Hollow cone (b) Flat fan (c) Solid stream (d) Rotary

Q38. Doubling the spraying pressure increases nozzle discharge approximately: (a) 2 times (b) 4 times (c) 1.41 times (d) No change

Q39. ULV spraying refers to application rates of: (a) <5 L/ha (b) <50 L/ha (c) <150 L/ha (d) <400 L/ha

Q40. The ratio used to express droplet spectrum uniformity is: (a) NMD/VMD (b) VMD/NMD (c) VMD × NMD (d) VMD − NMD

Q41. In mechanical agitation of sprayer tanks, paddles usually rotate at: (a) 10–50 rpm (b) 100–200 rpm (c) 500–800 rpm (d) 1000–1500 rpm

Q42. In hydraulic (jet) agitation, nozzles near the tank bottom are aimed: (a) Vertically up (b) About 30° above horizontal (c) Horizontally (d) 45° downward

Q43. Most efficient hydraulic agitation is obtained at: (a) High pressure (b) Low pressure (c) Medium pressure only (d) Independent of pressure

Q44. Which pump handles wettable-powder suspensions with least wear problems? (a) Gear pump (b) Diaphragm pump (c) Roller pump (d) Piston pump without liners

Q45. Droplet sizes most prone to spray drift are below about: (a) 400 μm (b) 250 μm (c) 100–150 μm (d) 600 μm

Q46. The pitch of knife sections and guards in a conventional mower cutter bar is: (a) 50.8 mm (b) 76.2 mm (c) 101.6 mm (d) 88.9 mm

Q47. “Registration” of a mower knife means: (a) Knife lead adjustment (b) Section centre coinciding with guard centre at stroke ends (c) Knife tension (d) Serial number stamping

Q48. Lead given to the mower cutter bar is about: (a) 2% of bar length (b) 10% of bar length (c) 25 mm regardless of length (d) None

Q49. Cutting in a cutter-bar mower occurs by: (a) Impact (b) Shear between section and ledger plate (c) Tearing (d) Bending

Q50. Threshing action in conventional threshers is mostly due to: (a) Rubbing only (b) Impact (c) Squeezing (d) Centrifugal force

Q51. The cylinder type predominant in modern grain combines is: (a) Spike tooth (b) Rasp bar (c) Wire loop (d) Hammer mill

Q52. Recommended cylinder peripheral speed for threshing wheat is about: (a) 5–10 m/s (b) 12–15 m/s (c) 25–30 m/s (d) 40–50 m/s

Q53. Cylinder–concave clearance in a thresher/combine is kept: (a) Uniform (b) More at entry, less at exit (c) Less at entry, more at exit (d) Zero at entry

Q54. In an axial-flow paddy thresher, the crop: (a) Passes straight through (b) Moves spirally along the cylinder axis (c) Falls by gravity only (d) Is threshed by rolls

Q55. Safe feeding-chute dimensions of power threshers are specified in: (a) IS 7898 (b) IS 9020 (c) IS 4468 (d) IS 6024

Q56. Reel index (reel speed ÷ forward speed) of a combine is normally: (a) 0.8–1.0 (b) 1.25–1.5 (c) 2.5–3.0 (d) 4.0–5.0

Q57. In a rectangular baler, twine is tied by the: (a) Plunger (b) Knotter (billhook mechanism) (c) Pickup (d) Star wheel

Q58. The cotton spindle picker harvests: (a) Whole plants (b) Only seed cotton from open bolls (selective) (c) Open + unopened bolls (d) Lint only

Q59. Chopper-type sugarcane harvesters deliver cane as: (a) Whole stalks (b) 25–30 cm billets (c) Bagasse (d) Shredded trash

Q60. In an elevator-type potato digger, soil separation is achieved by: (a) Blower (b) Agitated rod-link chain conveyor (c) Washing (d) Sieves and fan

## Answer Key

| Q | Ans | Q | Ans | Q | Ans | Q | Ans | Q | Ans | Q | Ans |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | c | 11 | b | 21 | b | 31 | b | 41 | b | 51 | b |
| 2 | b | 12 | b | 22 | b | 32 | b | 42 | b | 52 | c |
| 3 | c | 13 | b | 23 | c | 33 | b | 43 | b | 53 | b |
| 4 | b | 14 | c | 24 | b | 34 | b | 44 | b | 54 | b |
| 5 | b | 15 | b | 25 | c | 35 | b | 45 | c | 55 | b |
| 6 | b | 16 | b | 26 | b | 36 | b | 46 | b | 56 | b |
| 7 | b | 17 | c | 27 | b | 37 | b | 47 | b | 57 | b |
| 8 | c | 18 | c | 28 | b | 38 | c | 48 | a | 58 | b |
| 9 | c | 19 | c | 29 | c | 39 | a | 49 | b | 59 | b |
| 10 | c | 20 | b | 30 | b | 40 | b | 50 | b | 60 | b |

## Selected Explanations (high-yield)

- Q13/Q15: MB plough penetrates by suction; disc plough penetrates by weight — a classic contrast question.

- Q22: Draft is the horizontal, direction-of-travel component of pull.

- Q24: Kepner §11.6 — the fluted-wheel feed is “more positive in its metering action than the double run.”

- Q38: q ∝ √P ⇒ ×√2 ≈ 1.41.

- Q42/Q43: Kepner §21.16 — jets 1–2 in. above tank bottom, aimed 30° above horizontal; low pressures give most efficient agitation.

- Q46–Q49: standard cutter-bar geometry — 76.2 mm pitch; registration vs alignment (lead) are different adjustments; shear cutting.

- Q52: Kepner §17: 5000–6000 fpm ≈ 25–30 m/s for wheat.

- Q53: Converging clearance grips then progressively threshes without excess breakage.

## Suggested online resources for this unit

- NPTEL: “Farm Machinery” and “Tractors and Agricultural Machinery” video courses (IIT Kharagpur) — see digimat.in NPTEL mirror in your project links.

- ICAR e-courses: Farm Machinery & Equipment I and II (agrimoon/e-krishi shiksha PDFs).

- Testing standards & mechanization data: agriwelfare.gov.in (Mechanization & Technology Division), PIB releases on SMAM.

- Kepner, Bainer & Barger, chapters 7–9, 11–15, 17, 19–21 (in your folder).